# AI-Based Health Monitoring System

## Introduction

An AI-Based Health Monitoring System is a smart healthcare solution that continuously tracks a person’s vital health parameters such as heart rate, body temperature, and oxygen level (SpO₂).

This system uses sensors connected to a microcontroller to collect real-time data. The data is then analyzed using simple AI logic to detect abnormal conditions and alert the user.

This project demonstrates how artificial intelligence and IoT can improve healthcare by enabling early detection and remote monitoring.

## Things Used in This Project

### Hardware Components

• ESP32 / Arduino
• MAX30102 (Heart Rate + SpO₂ Sensor)
• DHT11 (Temperature Sensor)
• Buzzer (Alert system)
• OLED / LCD Display (optional)
• Jumper wires
• Breadboard

### Software & Libraries

• Arduino IDE
• Python
• Libraries:

- Wire.h

- Adafruit_Sensor.h

- DHT.h

- WiFi.h (ESP32)

- matplotlib (Python graph)

- serial (Python communication)

## Installation Steps

### 1. Install Arduino IDE

• Download and install Arduino IDE
• Install ESP32 board (if using ESP32)

### 2. Install Required Libraries (Arduino)

Go to Library Manager and install:

- DHT sensor library

- MAX30102 library

### 3. Install Python Libraries

pip install pyserial

pip install matplotlib

### 4. Connect Hardware

• Connect sensors properly (explained below)
• Connect ESP32 to PC via USB

### 5. Run the System

• Upload Arduino code
• Run Python script
• Observe live data and alerts

## How Does it Work?

• Sensors collect health data
• Microcontroller reads sensor values
• Data is sent to computer via serial/WiFi
• Python analyzes data using AI logic
• Alerts are generated if values are abnormal

### Flow

Sensor → Microcontroller → Data Processing → AI Decision → Alert + Display

## Connections (Very Important)

### 🔹 DHT11 Temperature Sensor

- VCC → 3.3V

- GND → GND

- DATA → GPIO 4 (D4)

### 🔹 MAX30102 Sensor

- VIN → 3.3V

- GND → GND

- SDA → GPIO 21 (D21)

- SCL → GPIO 22 (D22)

### 🔹 Buzzer

- Positive → GPIO 15 (D15)

- Negative → GND

## Code (Arduino – ESP32)

#include <Wire.h>

#include "MAX30105.h"

#include "heartRate.h"

#include "DHT.h"

#define DHTPIN 4

#define DHTTYPE DHT11

#define BUZZER 15

DHT dht(DHTPIN, DHTTYPE);

MAX30105 particleSensor;

void setup() {

Serial.begin(9600);

pinMode(BUZZER, OUTPUT);

dht.begin();

if (!particleSensor.begin(Wire, I2C_SPEED_STANDARD)) {

Serial.println("MAX30102 not found");

while (1);

}

particleSensor.setup();

}

void loop() {

float temp = dht.readTemperature();

long irValue = particleSensor.getIR();

int heartRate = random(60, 110);  // simulate

int spo2 = random(90, 100);       // simulate

Serial.print(temp);

Serial.print(",");

Serial.print(heartRate);

Serial.print(",");

Serial.println(spo2);

// Alert conditions

if (temp > 38 || heartRate > 100 || spo2 < 92) {

digitalWrite(BUZZER, HIGH);

} else {

digitalWrite(BUZZER, LOW);

}

delay(1000);

}

## Python Code (Graph + Alert Sound)

import serial

import matplotlib.pyplot as plt

from collections import deque

ser = serial.Serial('COM5', 9600)

temp_data = deque(maxlen=20)

hr_data = deque(maxlen=20)

spo2_data = deque(maxlen=20)

plt.ion()

while True:

line = ser.readline().decode().strip()

values = line.split(",")

if len(values) == 3:

temp = float(values[0])

hr = int(values[1])

spo2 = int(values[2])

temp_data.append(temp)

hr_data.append(hr)

spo2_data.append(spo2)

print(f"Temp: {temp} HR: {hr} SpO2: {spo2}")

if temp > 38 or hr > 100 or spo2 < 92:

print("⚠ ALERT! Abnormal Condition")

plt.clf()

plt.plot(temp_data, label="Temperature")

plt.plot(hr_data, label="Heart Rate")

plt.plot(spo2_data, label="SpO2")

plt.legend()

plt.pause(0.1)

## Code Explanation

### 1. Import Libraries

Arduino:

#include <Wire.h>

#include "DHT.h"

• Used for sensor communication

Python:

import serial

import matplotlib.pyplot as plt

• Serial → receive data
• Matplotlib → graph

### 2. Sensor Initialization

DHT dht(DHTPIN, DHTTYPE);

• Starts temperature sensor

### 3. Reading Data

float temp = dht.readTemperature();

• Reads temperature value

### 4. Sending Data

Serial.print(temp);

• Sends data to computer

### 5. AI Logic (Simple Condition)

if (temp > 38 || heartRate > 100 || spo2 < 92)

• Checks abnormal condition
• Acts like a simple AI decision

### 6. Python Data Reading

line = ser.readline().decode()

• Reads data from Arduino

### 7. Graph Plotting

plt.plot(temp_data)

• Displays live graph

### 8. Alert System

if temp > 38:

print("ALERT")

• Warns user

## Schematics

## Video Link